Arden Moss Consulting Engineers

Mark Bellamy
Project Manager
Castlegate Civils Ltd

Our ref: KVL-ADM-410-LT-S-0031
Date: 30 June 2025

Dear Mark,

River Kest Bridge - east abutment piles - response to RFI-0012

Thank you for RFI-0012 of 16 June 2025. We have reviewed the pile E3 boring log against borehole BH-E3 in the ground investigation report, which recorded a soft clay lens between -2.1 mAOD and -6.4 mAOD with mudstone at -9.8 mAOD.

The east abutment piles are to be lengthened from 18.0 m to 24.0 m, with the pile toe level lowered from -5.15 mAOD to -11.15 mAOD to found in the mudstone. The pile cut-off level is unchanged at +12.85 mAOD.

Temporary casing is to extend at least 1.0 m below the base of the soft clay. Revised drawings KVL-ADM-410-DR-S-0102 and KVL-ADM-410-DR-S-0110 will be issued at revision C02, with a new pile schedule. Brightwell Checking Services will re-check the revised design.

Yours sincerely,

Priya Nair, Lead Bridge Engineer, Arden Moss Consulting Engineers

cc: Helen Marsh, Project Manager, Northvale County Council; Anne Kowalski, Checking Engineer, Brightwell Checking Services