Castlegate Civils Ltd

# Submittal

| Project | Kestrel Valley Link Road |
| --- | --- |
| Document | KVL-CGC-410-SB-S-0004 |
| Revision | P01 |
| Status | AP - For approval |
| Date | 25 July 2025 |

| Title | Piling Method Statement - River Kest Bridge East Abutment |
| --- | --- |
| Submitted by | Ryan Doyle, Piling Supervisor, Castlegate Civils Ltd |
| For review by | Priya Nair, Lead Bridge Engineer, Arden Moss Consulting Engineers |
| Specification | Series 1600 - Piling and embedded retaining walls |

## 1 Scope

Construction of 8 No. 900 mm bored cast-in-place piles at the River Kest Bridge east abutment to drawings KVL-ADM-410-DR-S-0110 C02 and KVL-ADM-410-SC-S-0003 C02.

## 2 Method

Piles will be bored by rotary rig with temporary casing advanced through the alluvium and the soft clay lens. Casing will extend at least 1.0 m below the base of the soft clay, as confirmed on each boring log by the piling supervisor.

Concrete will be placed by tremie. Reinforcement cages will be fixed and inspected before placing.

## 3 Testing

Integrity testing on all piles. One static load test on a working pile nominated by the designer.