Castlegate Civils Ltd

# Submittal

| Project | Kestrel Valley Link Road |
| --- | --- |
| Document | KVL-CGC-320-SB-D-0009 |
| Revision | P01 |
| Status | AP - For approval |
| Date | 29 August 2025 |

| Title | Pond 2 Flow Control Chamber - Vortex Flow Control Device |
| --- | --- |
| Submitted by | Jo Kimani, Site Engineer, Castlegate Civils Ltd |
| For review by | Sam Whitfield, Drainage Engineer, Arden Moss Consulting Engineers |
| Specification | Series 500 - Drainage and service ducts |

## 1 Product

Aqualine VFC-150 vortex flow control device in a 1500 mm precast chamber at the Pond 2 East outlet, set to pass 5.0 l/s at a design head of 1.8 m.

## 2 Performance

The manufacturer's head-discharge curve is attached. Discharge at 1.8 m head is 4.9 l/s. A bypass door and a sump are included for maintenance.