Arden Moss Consulting Engineers

Mark Bellamy
Project Manager
Castlegate Civils Ltd

Our ref: KVL-ADM-330-LT-S-0034
Date: 15 July 2025

Dear Mark,

Mill Lane culvert - response to RFI-0013

Further to RFI-0013 and Northvale Water's letter KVL-NVW-610-LT-U-0003, the culvert invert is to be lowered by 450 mm from +10.80 mAOD to +10.35 mAOD. This gives 0.75 m clearance between the culvert roof and the water main.

Because the main's cover under the carriageway is only 0.9 m, a 250 mm reinforced concrete protection slab, 6.0 m long, is to be provided over the main.

Revised culvert drawing KVL-ADM-330-DR-S-0205 C02 and slab details KVL-ADM-330-DR-S-0206 will be issued by 20 August 2025, after re-check by Brightwell Checking Services.

Yours sincerely,

Priya Nair, Lead Bridge Engineer, Arden Moss Consulting Engineers

cc: Tom Reddy, Highways Engineer, Northvale County Council; Gareth Lloyd, Asset Engineer, Northvale Water