Northvale County Council

Gareth Lloyd
Asset Engineer
Northvale Water

Our ref: KVL-NCC-610-LT-U-0014
Date: 19 June 2025

Dear Gareth,

Kestrel Valley Link Road - Mill Lane water main - trial hole results

Castlegate Civils excavated three trial holes (TH1 to TH3) over your 300 mm ductile iron water main at Mill Lane between 9 and 11 June 2025. The main was found at an invert level of about +13.40 mAOD, some 0.6 m deeper than your records show (+14.00).

At this level the main is only 0.30 m above the roof of the new Mill Lane box culvert, and its cover below the new carriageway will be about 0.9 m.

Please confirm your clearance and cover requirements, and whether you would accept a protection slab or require the main to be diverted.

Yours sincerely,

Tom Reddy, Highways Engineer, Northvale County Council

cc: Priya Nair, Lead Bridge Engineer, Arden Moss Consulting Engineers; Mark Bellamy, Project Manager, Castlegate Civils Ltd